GM RAILING® is a fully prefabricated frameless-glass balustrade system. GM RAILING® Side is designed to be side mounting to the people-facing side of a solid base wall or attic. The glass is designed to be clamped along its full lower edge and meets loading and Building Regulations.

*: delete as appropriate.

| NAME/POS | TEXT / QUANTITY / UNIT |  | Unit Price (UP) | Total Price (TP) |
| --- | --- | --- | --- | --- |
| Name 42.0 | FRAMELESS-GLASS RAILING |  |  |  |
| 42.1.00 | General Description |  |  |  |
|  | Tech. Advantages: | Frameless, single-edge supported fixed glass balustrade with evenly distributed load bearing fixings. There is no unevenly distributed point loading of the glass (wedges directly on the glass) or clamps. The method of fixing the glass protects the laminated glass against early delamination. The system is fully tested as a complete fabricated unit to building standards. |  |  |
|  | Design: | The substructure is manufactured in extruded aluminium with an anodized finish. An adjustable toothed plate is incorporated to enable various fixing positions, for example, side mounted to the people-facing side of a solid base wall or attic. Profiles are available up to 6 metres in length. Allowance needs to be made for expansion due to heat. |  |  |
|  | Requirements: | private use* / public use* Fall height < 12 m* / fall height > 12 m* Interior use* / exterior use* (Wind load ………..kN / m²) |  |  |
|  | Glass specification: | Laminated safety glass (LSG) made of float glass, SSG or HSG, based upon loading requirements. The interlayer is clear PVB film – min. thickness 1,52 mm. All visible glass edges are polished. There are no holes drilled into the glass for fixing. The glass is bonded into the supporting anodised aluminium base profile. Vertical fine adjustment of the glass is possible to (± 20 mm). Rapid replacement of panels is also possible due to the system being manufactured in pre-assembled component parts. All bonding is certified according to the specification criteria. |  |  |
|  | Glass type: | clear* / flint glass* / matt film* / ICE-H* / ……………. |  |  |
|  | Cover profile: | exterior lining angle profile ……../……../………mm* Surface: EV1 natural anodised* / powder-coat. RAL ………. * / mill finish* High-grade stainless steel edge part K240* |  |  |
|  | Handrail: | High-grade stainless steel U profile K240* / Wood-covered* round* / oval* / angular* Wood type: beech steamed* / oak * / europ. maple* Surface: stained* / oiled* / varnished* High-grade stainless steel grooved tube* round* / oval* Surface: K 240 ground* / polished* Aluminium grooved tube, round* Surface: EV1 natural anodised* /…………………… |  |  |
|  | Product: | GM RAILING® |  |  |
|  | Series: | GM RAILING® Front AIO* (h2 = 140mm), Front AIT* (h2 = variable) |  |  |
|  | Producer: | Glas Marte GmbH, Abt. GM Railing, www.gm-railing.com; info@gm-railing.com |  |  |
|  |  | or equal quality. |  |  |
|  | Offered product: ________________________________ |  |  |  |
| NAME/POS | TEXT / QUANTITY / UNIT |  | Unit Price (UP) | Total Price (TP) |
|  |  |  | carry-over .………...………. |  |
| 42.1.10 | Loadings, Dimensions and Planning |  |  |  |
|  | The system is supplied with full compliance to Building Regulations and supported with test certificates and structural calculations for submission to the Building control Officer/Inspector in accordance with EN 12600 based on original test criteria. The system is also supplied with full instructions for installation and material specification, spare parts list and maintenance instruction, together with manufacturer declaration of product compliance are provided. |  |  |  |
|  | Overall technical design |  | UP ……………… | TP ……………….. |
| 42.1.20 | Horizontal Frameless-Glass Railing |  |  |  |
|  | Full-glass railing according to description Component: _________________________ Level: _________________________ see diagram no.: _________________________ Railing height (hb) from upper edge of finished floor line Glass stick-out (h1) from upper edge of profile Number of corners Regular length of individual glass construction modules approx. …..……...m until ………………m |  | ………….….…m ………….….…m ………..…piece |  |
|  | Total.................... running metres |  | UP ……………… | TP ……………….. |
| 42.1.30 | Inclined/raked Frameless-Glass Railing for Ramps |  |  |  |
|  | Full-glass railing according to description incl. platforms Component: _________________________ Level: _________________________ see diagram no.: _________________________ Railing height (hb) from upper edge of finished floor line Glass stick-out (h1) from upper edge of profile Number of corners Regular length of individual glass construction modules approx. …..……...m until ………………m (running metre indication corresponds to handrail length; glass formats normally parallelograms, platform lengths > 3m are calculated horizontally, heights vertically) |  | ………….….…m ………….….…m ….….…...…units |  |
|  | Total.................... running metres |  | UP ……………… | TP ……………….. |
| 42.1.40 | Handrail |  |  |  |
|  | Handrail design in-line with specification and description horizontal and inclined bonded with silicone (no dry profiles) to glass edge on site. |  |  |  |
|  | Total.................... running metres |  | UP ……………… | TP ……………….. |

| NAME/POS | TEXT / QUANTITY / UNIT |  | Unit Price (UP) | Total Price (TP) |
| --- | --- | --- | --- | --- |
|  |  |  | carry-over ………...………. |  |
| 42.1.41 | Glass Stripe Edge Protection |  |  |  |
|  | Instead of the handrail, a 22x6mm profile glass stripe is available with chamfered and fine-ground edges, single lengths < 3m, made of borosilicate glass. This is applied to the upper glass edge in the factory as an edge protection. Based on loading requirements, the glass type and glass thickness must be considered when using this solution. |  |  |  |
|  | Total.................... running metres |  | UP ……………… | TP ……………….. * Requirement* |
| 42.1.60 | Handrail Connections |  |  |  |
|  | Connections made of ground high-grade steel K240 |  |  |  |
|  | Handrail connection to wall .…….units |  | ……………………. | …………………….. |
|  | 90° edge connections .…….units |  | ……………………. | …………………….. |
|  | Longitudinal connections .…….units |  | ……………………. | …………………….. |
|  | End upright .…….units |  | ……………………. | …………………….. |
|  | Connections total |  | UP ……………… | TP ……………….. |
| 42.1.70 | Fixation to the Building Structure |  |  |  |
|  | Dowels , screws and fasteners, high-grade steel for exterior use, approved products only. |  |  |  |
|  | Total.................... running metres |  | UP ……………… | TP ……………….. |
|  | Total Name 42.0 FRAMELESS-GLASS RAILING |  |  | € |
|  | + 20 % VAT |  |  | € |
|  | Total price, gross |  |  | € |